Simulation of beamforming gain under steering angle, antenna elements, frequency,SNR, and interference

AIM: Simulation of beamforming gain under steering angle, antenna elements, frequency,

SNR, and interference

Objective

Simulate the beamforming gain of an antenna array by varying:

- Steering angle (θ)

- Number of antenna elements (N)

- Operating frequency (f)

- Signal-to-Noise Ratio (SNR)

- Interference level

clc;

clear;

close all;

N = [4 8 16];

theta = -90:1:90;

figure;

for k = 1:length(N)

n = N(k);

AF = zeros(size(theta));

for i = 1:length(theta)

psi = pi*sind(theta(i));

AF(i) = abs(sum(exp(1j*(0:n-1)*psi)));

end

AF = AF/max(AF);

subplot(3,1,k)

plot(theta,20*log10(AF),'LineWidth',2);

grid on;

xlabel('Steering Angle (degrees)');

ylabel('Gain (dB)');

title(['Beamforming Gain for N = ',num2str(n)]);

ylim([-40 0]);

end

Expected Results

- Increasing antenna elements (N) increases beamforming gain and narrows the main beam.

- Changing the steering angle shifts the beam direction.

- Higher frequency provides a narrower beam (for the same array spacing in wavelengths).

- Increasing SNR improves signal quality and beamforming performance.

- Strong interference reduces received signal quality, but beamforming suppresses interference from unwanted directions.